import serial #for serial communication with GSM SIM800L

import time

import pygame #to play music

# _____________________________________________________________________________#

# Intro text

print("Setting up Raspberry PI IVR")

#Speak with SIM800 -> gets AT command return as response

def SIM800(command):

AT_command = command + "\r\n"

ser.write(str(AT_command).encode('ascii'))

time.sleep(1)

if ser.inWaiting() > 0:

echo = ser.readline() #waste the echo

response_byte = ser.readline()

response_str = response_byte.decode('ascii')

return (response_str)

else:

return ("ERROR")

#checks if SIM800L is speaking and returns it as response

def wait_for_SIM800():

echo = ser.readline()  # waste the echo

response_byte = ser.readline()

response_str = response_byte.decode('ascii')

return (response_str)

#Checks SIM800L status and connects with TrueView

def Init_GSM():

if "OK" in SIM800("AT"):

if ("OK" in (SIM800("AT+CLCC=1"))) and ("OK" in (SIM800("AT+DDET=1"))) and ("OK" in (SIM800("AT+CNMI =0,0,0,0,0"))) and ("OK" in (SIM800("AT+CMGF=1"))) and ("OK" in (SIM800("AT+CSMP=17,167,0,0"))):  # enble DTMF / disable notifications

print("SIM800 Module -> Active and Ready")

else:

print("------->ERROR -> SIM800 Module not found")

#plays the given wav file #8000Mhz mono audio WAV works best on SIM800L

def play_wav(file_name):

pygame.mixer.init(8000)

pygame.mixer.music.load(file_name)

pygame.mixer.music.play()

#while pygame.mixer.music.get_busy() == True:

#continue

# Makes a call to given number and returns NONE, NOT_REACHABLE, CALL_REJECTED, REJECTED_AFTER_ANSWERING,  REQ_CALLBACK,CANCELED, CONFIRMED

def Call_response_for (phone_number):

AT_call = "ATD" + phone_number + ";"

response = "NONE"

time.sleep(1)

ser.flushInput() #clear serial data in buffer if any

if ("OK" in (SIM800(AT_call))) and (",2," in (wait_for_SIM800())) and (",3," in (wait_for_SIM800())):

print("RINGING...->", phone_number)

call_status = wait_for_SIM800()

if "1,0,0,0,0" in call_status:

print("**ANSWERED**")

ser.flushInput()

play_wav("intro.wav")

time.sleep(0.5)

dtmf_response = "start_over"

while dtmf_response == "start_over":

play_wav("press_request.wav")

time.sleep(1)

dtmf_response = wait_for_SIM800()

if "+DTMF: 1" in dtmf_response:

play_wav("confirmed.wav")

response = "CONFIRMED"

hang = SIM800("ATH")

break

if "+DTMF: 2" in dtmf_response:

play_wav("canceled.wav")

response = "CANCELED"

hang = SIM800("ATH")

break

if "+DTMF: 9" in dtmf_response:

play_wav("callback_response.wav")

response = "REQ_CALLBACK"

hang = SIM800("ATH")

break

if "+DTMF: 0" in dtmf_response:

dtmf_response = "start_over"

continue

if "+DTMF: " in dtmf_response:

play_wav("invalid_input.wav")

dtmf_response = "start_over"

continue

else:

response = "REJECTED_AFTER_ANSWERING"

break

else:

#print("REJECTED")

response = "CALL_REJECTED"

hang = SIM800("ATH")

time.sleep(1)

#ser.flushInput()

else:

#print("NOT_REACHABLE")

response = "NOT_REACHABLE"

hang = SIM800("ATH")

time.sleep(1)

#ser.flushInput()

ser.flushInput()

return (response)

#Receives the message and phone number and send that message to that phone number

def send_message(message, recipient):

ser.write(b'AT+CMGS="' + recipient.encode() + b'"\r')

time.sleep(0.5)

ser.write(message.encode() + b"\r")

time.sleep(0.5)

ser.write(bytes([26]))

time.sleep(0.5)

print ("Message sent to customer")

time.sleep(2)

ser.flushInput()  # clear serial data in buffer if any

def incoming_call():

while ser.in_waiting: #if I have something in the serial monitor

print ("%%Wait got something in the buffer")

ser.flushInput()

response = SIM800("ATH") #cut the incoming call

if "+CLCC" in response:

cus_phone = response[21:31]

print("%%Incoming Phone call detect from ->", cus_phone)

return (cus_phone)

else:

print("Nope its something else")

return "0"

return "0"

cus_name = "Aisha"

cus_phone = "96883XXXXX"

while (1): #Infinite loop

# COM defanition for windows -> Should change for Pi

ser = serial.Serial("/dev/ttyUSB0", baudrate=9600, timeout=15)  # timeout affects call duration and waiting for response currently 30sec

print("Established communication with", ser.name)

Init_GSM() #check if GSM is connected and initialize it

print("_____________________IVR START___________________")

response = Call_response_for(cus_phone) #place a call and get response from customer

print ("Response from customer => ", response)

if response == "CONFIRMED":

text_message = "Hi " + cus_name + ". Your booking has been confirmed. Thank you!!. -TrueView"

send_message(text_message, cus_phone)

if response == "CANCELED":  # if the response was to cancel

text_message = "Hi " + cus_name + ". Sorry that you have decided to cancel your booking. If you cancled by mistake, kindly contact us through phone. - TrueView "

send_message(text_message, cus_phone)

if ((response == "CALL_REJECTED") or (response == "REJECTED_AFTER_ANSWERING")):  # if the response was rejected

text_message = "Hi " + cus_name + ". We from TrueView.com have been trying to reach you, to confirm your booking. You will receive another call within few minutes, we kindly request you to answer it. Thank you"

send_message(text_message, cus_phone)

print("_____________________IVR END___________________")

ser.close()

time.sleep (5)